Academic Editor: Bruce Henry Received: 2 September 2025 Revised: 3 October 2025 Accepted: 14 October 2025 Published: 18 October 2025 Citation: Luan, F.; Yang, J.; Zhang, H. MDF-YOLO: A Hölder-Based Regularity-Guided Multi-Domain Fusion Detection Model for Indoor Objects. Fractal Fract. 2025, 9, 673. https://doi.org/10.3390/ fractalfract9100673 Copyright: © 2025 by the authors. Licensee MDPI, Basel, Switzerland. This article is an open access article distributed under the terms and conditions of the Creative Commons Attribution (CC BY) license (https://creativecommons.org/ licenses/by/4.0/). Article MDF-YOLO: A Hölder-Based Regularity-Guided Multi-Domain Fusion Detection Model for Indoor Objects Fengkai Luan , Jiaxing Yang and Hu Zhang  School of Information Engineering, Wuhan University of Technology, Wuhan 430070, China  Correspondence: huzhang@whut.edu.cn Abstract With the rise of embodied agents and indoor service robots, object detection has become a critical component supporting semantic mapping, path planning, and human–robot inter- action. However, indoor scenes often face challenges such as severe occlusion, large-scale variations, small and densely packed objects, and complex textures, making existing meth- ods struggle in terms of both robustness and accuracy. This paper proposes MDF-YOLO, a multi-domain fusion detection framework based on Hölder regularity guidance. In the backbone, neck, and feature recovery stages, the framework introduces the CrossGrid Mem- ory Block, Hölder-Based Regularity Guidance–Hierarchical Context Aggregation module, and Frequency-Guided Residual Block, achieving complementary feature modeling across the state space, spatial domain, and frequency domain. In particular, the HG-HCA module uses the Hölder regularity map as a guiding signal to balance the dynamic equilibrium between the macro and micro paths, thus achieving adaptive coordination between global consistency and local discriminability. Experimental results show that MDF-YOLO signif- icantly outperforms mainstream detectors in metrics such as mAP@0.5, mAP@0.75, and mAP@0.5:0.95, achieving values of 0.7158, 0.6117, and 0.5814, respectively, while main- taining near real-time inference efficiency in terms of FPS and latency. Ablation studies further validate the independent and synergistic contributions of CGMB, HG-HCA, and FGRB in improving small-object detection, occlusion handling, and cross-scale robustness. This study demonstrates the potential of Hölder regularity and multi-domain fusion mod- eling in object detection, offering new insights for efficient visual modeling in complex indoor environments. Keywords: indoor object detection; Hölder regularity; multi-domain fusion; context aggregation; frequency-domain detail recovery 1. Introduction With the rapid progress of artificial intelligence and robotics, embodied agents— intelligent systems capable of autonomously perceiving their surroundings, making de- cisions, and executing actions are increasingly integrated into daily life [ 1,2]. In indoor environments, devices such as service robots and cleaning robots are widely deployed, relying heavily on accurate visual perception to support tasks such as autonomous navi- gation, human–robot interaction, and scene understanding [3,4]. Within this framework, object detection plays a central role by identifying and localizing objects, thereby providing critical spatial and semantic information for downstream modules including semantic map construction [5,6], path planning [7,8], and task-level decision-making [9]. Thus, develop- Fractal Fract. 2025, 9, 673 https://doi.org/10.3390/fractalfract9100673

Fractal Fract. 2025, 9, 673 2 of 31 ing indoor object detection algorithms that are both highly accurate and robust is crucial for improving the overall reliability and effectiveness of embodied agents. Nevertheless, the inherent complexity of indoor environments poses substantial chal- lenges for object detection [10]. First, objects are often subject to severe occlusion and clutter, fragmenting their features and demanding strong global reasoning ability from models [11]. Second, the dramatic scale variation from large pieces of furniture to small tabletop items requires effective multi-scale representation and fusion [12]. Third, the recognition of small objects depends heavily on high-frequency details such as edges and textures, which are frequently degraded or lost during the down-sampling process of deep convolutional networks, leading to missed or false detections [13]. Consequently, addressing global con- textual reasoning, scale adaptation, and fine-detail preservation simultaneously remains the central difficulty in indoor object detection. To address the challenges of general-purpose object detection, detectors centered around Convolutional Neural Networks (CNNs) have undergone continuous evolution over the past decade [ 14]. Among these, single-stage detectors have become dominant in real-time applications due to their streamlined and efficient design. The trajectory of their research and development has consistently sought a balance between accuracy and efficiency. This evolution spans from YOLO, which unified detection as a regression problem [15], and RetinaNet, which introduced Focal Loss to effectively resolve the extreme imbalance between positive and negative samples [16], to the rise of anchor-free methods like FCOS [17]. The performance and paradigms of single-stage detectors have been per- petually innovating. Concurrently, driven by the demand for mobile and edge computing, lightweight backbones such as MobileNet and ShuffleNet have been extensively researched and applied [18], giving rise to detectors like EfficientDet, which exhibit outstanding perfor- mance on resource-constrained platforms [19]. Despite their widespread success in object detection, CNNs inherently operate within limited receptive fields, introducing the local in- ductive bias [20]. This means that CNNs are highly effective at capturing fine-grained local features such as edges and textures, but they struggle to model long-range dependencies between distant regions in an image—for example, recognizing the association between a table surface and its legs when they appear far apart. While this bias improves efficiency in learning local patterns, it hampers the ability of CNN-based detectors to perform holistic reasoning in complex indoor scenes, thereby reducing their robustness to severe occlusion and scale variation. To overcome the locality bottleneck of CNNs, vision Transformer architectures, epit- omized by ViT, have emerged. Their core self-attention mechanism is highly effective at capturing global context. Building upon this, the DETR framework reformulated object detection as an end-to-end set prediction problem [21]. However, the powerful global mod- eling capability of Transformers comes at a significant computational cost. The complexity of their self-attention mechanism grows quadratically with input resolution, leading to prohibitive training and inference overheads at high resolutions [22], making it difficult to meet the stringent real-time requirements of indoor applications. In response to the aforementioned challenges, current research trajectories primarily encompass several key directions. On one hand, researchers are dedicated to exploring more efficient mechanisms for modeling long-range dependencies. These range from various novel attention modules to the recently prominent State Space Models (SSMs) in the vision domain [ 23,24], all aiming to achieve global contextual awareness while maintaining linear computational complexity. On the other hand, the academic community is investigating more sophisticated multi-scale feature interaction and fusion modules. This includes enhancing key feature representations through techniques like attention mechanisms [25] or incorporating multi-modal information, such as 3D point clouds, to

Fractal Fract. 2025, 9, 673 3 of 31 aid in understanding complex spatial relationships [ 26]. Furthermore, approaching the problem from novel perspectives, such as the frequency domain, by explicitly compensating for features to recover details lost during network propagation, has also become an effective strategy for improving the performance of small object detection [27]. Based on the preceding analysis, this paper proposes MDF-YOLO, a multi-domain fusion object detector tailored for indoor scenes. Our objective is to systematically address the core challenges of indoor detection through a synergistically optimized solution that achieves performance breakthroughs in global context modeling, multi-scale fusion, and detail recovery, all while preserving real-time efficiency. The primary contributions of this paper are as follows: 1. We design a novel backbone network module named the CrossGrid Memory Block (CGMB). This module integrates state-space modeling and local convolutional path- ways in parallel, capturing global context and long-range spatial dependencies with linear complexity via an orthogonal grid memory mechanism, thereby effectively addressing issues of severe occlusion. 2. We propose the Hölder-Based Regularity Guidance-Hierarchical Context Aggregation (HG-HCA) module. This module integrates macro- and micro-context pathways under the guidance of a Hölder-based regularity prior, where a differentiable regularity map is computed to characterize local smoothness and roughness. Through a lightweight calibration mechanism, the regularity map is transformed into task-oriented per-pixel guidance signals. This design enables the network to dynamically balance global structural consistency and local discriminative detail, thereby improving robustness against large-scale variations, complex textures, and dense occlusion in indoor scenes. 3. We introduce an up-sampling module named the Frequency-Guided Residual Block (FGRB). This module augments the spatial up-sampling path with a parallel frequency- domain compensation path. It enhances high-frequency components using a learnable frequency weight matrix and restores image details in a residual manner, thereby improving detection performance for small objects. Extensive experimental results on a public indoor scene dataset demonstrate that MDF-YOLO surpasses existing mainstream methods in both accuracy and efficiency, val- idating the effectiveness and novelty of the proposed multi-domain fusion-based object detection network. 2. Related Work 2.1. T raditional Rule-Based and Handcrafted Feature Methods Prior to the widespread adoption of deep learning, research in indoor object detec- tion was predominantly characterized by sensor-driven geometric rule-based methods and computer vision approaches based on handcrafted features. Sensor-based methods utilized geometric information acquired from ultrasound, infrared, and laser rangefinders to ensure motion safety through local avoidance and path correction. The ultrasonic obsta- cle avoidance framework by Borenstein and Koren achieved stable avoidance under low computational constraints through continuous rangefinding and wall-following strategies, suppressing oscillatory behaviors in “trap regions” via vector field synthesis [28]. The oc- cupancy grid map proposed by Moravec and Elfes introduced a grid-based environmental representation with a Bayesian update mechanism, enabling recursive estimation of spatial occupancy probabilities under noisy rangefinding conditions and providing a unified representation for indoor mapping and obstacle perception [29]. The probabilistic robotics framework by Thrun et al. integrated localization, mapping, and decision-making into a unified Bayesian inference framework, maintaining global consistency amidst multi-sensor noise and dynamic occlusions, thereby supporting early indoor mobile robots in reasoning

Fractal Fract. 2025, 9, 673 4 of 31 about spatial accessibility and safety [30]. This line of research offered the advantages of high real-time performance and implementation simplicity at the geometric level, but it lacked the capability to characterize object categories, semantic attributes, and fine-grained boundaries, thus failing to meet the demands of subsequent indoor detection tasks. As image sensors and computational platforms matured, the research focus shifted towards a detection paradigm composed of manually designed visual features and shallow discriminative classifiers. Lowe’s SIFT performed keypoint detection in scale-space and represented appearance using orientation-aligned local gradient histograms, demonstrat- ing robustness to viewpoint and illumination changes and establishing the foundation for recognition based on local invariant features [ 31]. Bay et al.’s SURF approximated Hessian responses using integral images and box filters, substantially reducing descriptor computation costs and significantly improving real-time performance while maintaining robustness [32]. The cascaded Boosting framework by Viola and Jones utilized Haar-like features and a cascade of weak classifiers to achieve high-speed sliding-window detec- tion, attaining near real-time performance on rigid objects like faces and propelling early embedded vision applications [ 33]. Dalal and Triggs’ HOG eliminated illumination ef- fects through gradient statistics normalized within cells and blocks, enhancing pedestrian detection recall and precision via a linear SVM and becoming a standard configuration for sliding-window detection [ 34]. The Deformable Part Model by Felzenszwalb et al. performed discriminative training under geometric deformation constraints between a root template and part templates, significantly enhancing detection performance for non-rigid objects and under occlusion, and showing strong adaptability to intra-class variations in complex indoor layouts [ 35]. In subsequent work, Dalal, Triggs, and Schmid jointly modeled appearance gradients and optical flow orientation histograms, which markedly improved the robustness of human detection in video sequences under dynamic scenes, showcasing the complementary benefits of appearance and motion cues [36]. This approach could provide interpretable mid-level representations and robust discriminative boundaries on indoor images, but it still faced significant bottlenecks in cross-domain generalization, complex occlusion, and extreme scale variations. The popularization of RGB-D sensors led to the widespread adoption of 3D geometric features in indoor object recognition and localization, with traditional methods representing and matching targets using local surface geometry or multi-modal templates. The Spin Image by Johnson and Hebert formed a 2D projection histogram from surface normals and projection distances, achieving high-recall matching of 3D parts in cluttered and partially occluded scenes and providing a reusable descriptor for rigid instance recognition in indoor environments [37]. Rusu et al.’s FPFH computed fast normal-related statistics within the k-neighborhood of a point cloud and reduced coupling term overhead, achieving an excellent efficiency-accuracy trade-off in large-scale point cloud feature matching and pose estimation tasks, well-suited for the real-time requirements of indoor recognition and registration [38]. Hinterstoisser et al.’s LINEMOD employed multi-modal template matching based on gradient orientation and surface normals to achieve rapid detection and pose estimation of texture-less objects. It maintained a stable detection rate in indoor scenes with strong occlusion, high reflectance, and cluttered backgrounds, demonstrating practical engineering applicability [39]. These 3D and multi-modal traditional methods possess unique advantages in structure recovery and pose estimation, but they still rely on meticulous design and extensive parameter tuning for cross-view generalization, template scale management, and inter-class separability. Traditional methods based on rules and handcrafted features established a technologi- cal trajectory for indoor object detection, progressing from geometric obstacle avoidance to semantic discrimination and 3D matching, and could achieve reliable performance under

Fractal Fract. 2025, 9, 673 5 of 31 controlled conditions and for specific targets. However, these methods, relying on human priors and fixed-structure representations, are prone to performance degradation when confronted with occlusion, multi-scale challenges, and appearance diversity in indoor environments, which prompted a research shift towards detection paradigms that are automatically learnable and end-to-end optimizable. 2.2. General-Purpose Deep Learning Models Deep learning has catalyzed a paradigm shift in object detection from handcrafted features and shallow classifiers to end-to-end learnable representations. R-CNN combined region proposal generation with CNN feature learning, significantly elevating detection accuracy on general-purpose datasets and establishing a baseline for deep feature-centric detection [40]. SPP-Net extracted regional features from a shared feature map via spatial pyramid pooling, circumventing the expense of redundant forward passes over the entire image; however, its training pipeline was not fully end-to-end, a concept that laid the groundwork for subsequent unified detection frameworks [41]. Fast R-CNN introduced RoI Pooling and a multi-task loss to jointly optimize bounding box classification and regression on a shared feature map, realizing efficient end-to-end training (with the exception of region proposal generation) and striking a better balance between accuracy and efficiency [ 42]. Faster R-CNN, through the synergy of a Region Proposal Network and a detection network, integrated region proposal generation into the main network, thus unifying the two-stage detection paradigm and enabling joint training, which solidified its mainstream position for high-accuracy detection [43]. Mask R-CNN augmented the two-stage architecture with an additional branch for simultaneous instance segmentation, improving spatial localization precision with an aligned RoI operation while maintaining excellent extensibility [44]. In the pursuit of real-time performance, single-stage detectors replaced the region proposal mechanism with dense prediction. SSD performed direct classification and re- gression on multi-scale feature maps, balancing speed and accuracy to become a common baseline for mobile deployments [45]. The YOLO series formulated detection as a unified regression problem and, based on this framework, significantly improved stability and accuracy through enhancements such as anchor boxes and batch normalization [ 46]. It strengthened its coverage of differently sized objects via multi-scale prediction and deeper backbone networks [47], with subsequent work further refining feature aggregation and training strategies to enhance robustness and usability [48]. The issue of extreme positive– negative sample imbalance in single-stage frameworks was mitigated by the Focal Loss, which improved detection performance on small and hard examples while maintaining high efficiency [16]. In terms of model scalability, EfficientDet employed a compound scaling strategy coupled with a high-efficiency bidirectional feature pyramid, achieving an integrated design scalable from resource-constrained devices to server-side applications and demonstrating outstanding performance in the accuracy–efficiency trade-off [49]. General-purpose detectors have achieved a significant leap in performance over tra- ditional methods on public datasets, yet their design philosophy is often geared towards generic scenarios. In indoor environments, the local inductive bias of convolution limits the ability for global reasoning on severely occluded and cluttered objects. Down-sampling and high-level abstraction lead to the attenuation of high-frequency edges and textures during propagation, thereby impairing the discriminability of small objects. Although multi-scale feature fusion can alleviate the semantic gap caused by scale variations, it re- mains insufficient for the extreme scale distributions found indoors, from large furniture to minute items. These issues have become recognized challenges in indoor detection, driving research to undertake targeted explorations in the directions of global context modeling, cross-scale fusion, and detail recovery [12,13].

Fractal Fract. 2025, 9, 673 6 of 31 2.3. Explorations T argeting Challenges in Indoor Scenes While general-purpose detectors have established a strong technical baseline, their bot- tlenecks in indoor applications have become increasingly apparent. Three issues dominate in this setting—severe occlusion, extreme scale variation, and the loss of fine details—which have in turn stimulated extensive research along three directions: global context modeling, cross-scale semantic alignment, and detail restoration. These lines of work offer diverse technical paths for tackling the key difficulties of indoor scenes, yet they also reveal several common limitations in terms of efficiency, stability, and deployability. In global context modeling under occlusion and crowding, relational reasoning and structural priors are widely used to compensate for the locality of convolutions. Relation Networks reweight the relationships among candidate regions to alleviate semantic loss and improve localization consistency, but region-level relation graphs incur non-negligible computational and memory overhead in dense scenes and are sensitive to the quality of candidate generation [50]. Occlusion-aware R-CNN combines aggregation losses with part visibility modeling to reduce missed detections under group occlusions; however, part-level annotations and visibility labels increase data production costs, and robustness remains limited under cross-category transfer [ 51]. Context-Aware CompositionalNets enhance interpretability under occlusion through part–whole compositional representations, yet they rely on relatively stable structural priors and adapt less effectively to non-rigid objects or categories with large appearance variation [52]. At the post-processing level, Soft-NMS replaces hard suppression with continuous score decay to improve recall and precision in crowded scenes, but it cannot fundamentally remedy the insufficient characterization of long-range dependencies at the feature level [53]. Overall, this line of work has direct value for complex indoor stacking, but introducing efficient long-range dependencies while preserving 2D topology remains a central challenge. In cross-scale variation and semantic alignment, feature pyramids and scale-aware training continue to evolve to cover the span from large furniture to tiny devices. ASFF learns spatially adaptive fusion weights to ease spatial misalignment across feature levels and can enhance scale robustness with low additional cost, yet it still tends to favor particular levels under extremely long-tailed scale distributions [54]. NAS-FPN discovers efficient topologies in the space of cross-level connections via neural architecture search, unifying top-down and bottom-up information flows; however, search costs and hardware dependencies are substantial, and practical deployment requires rebalancing complexity and gains across platforms [55]. TridentNet employs multi-branch convolutional paths with different receptive fields and scale-aware training to improve stability at extreme scales; although branches can be pruned at inference to control cost, multi-branch training still brings significant memory and optimization burdens [56]. At the proposal stage, Guided Anchoring reduces invalid candidates with learnable priors and position guidance and is more friendly to objects with diverse aspect ratios, but dependence on anchor assumptions and data distributions persists [57]. In summary, efficient and stable cross-level semantic alignment remains challenging on indoor datasets with long-tailed scales and diverse shapes, and the balance between complexity and benefit is not consistent across resolutions and hardware. In detail preservation and small-object visibility, upsampling operators and frequency- domain attention are used to restore high-frequency textures and edges, and they com- plement geometric modalities. CARAFE adopts content-aware reassembly kernels that are more favorable to boundaries and textures than fixed interpolation or transposed convolution, but the extra operators and buffering required for kernel prediction must be carefully evaluated for real-time scenarios [58]. FcaNet generalizes channel attention compression to the frequency domain and introduces multi-spectrum attention to enhance

Fractal Fract. 2025, 9, 673 7 of 31 high-frequency responses for detail-sensitive categories; however, alignment between fre- quency enhancement and task features still relies on heuristic design, and controlling edge artifacts and ringing effects requires additional regularization and implementation care [59]. Along the multimodal route, VoteNet and its extensions perform 3D detection via spatial voting in point clouds and hybrid geometric primitives; ImVoteNet aligns image-space votes with camera parameters to improve detection of occluded and small objects; H3DNet and 3DETR demonstrate competitiveness under 3D priors and end-to-end Transformer designs [60–63]; Deep Sliding Shapes earlier established a feasible paradigm for amodal 3D object detection in RGB-D scenes through voxelized 3D convolutions and a 3D RPN [64]. This route shows clear advantages under indoor conditions such as low light, reflective materials, and near-field occlusion, yet in practice it imposes higher requirements on depth- sensor availability, cross-modal calibration, and inference latency, limiting deployment on some platforms. Beyond the above threads, explicit modeling of local regularity has attracted growing attention. Differentiable measures of local regularity can provide a complementary sig- nal to saliency or attention mechanisms by distinguishing texture-smooth regions from edge/detail-rich regions, and by offering pixel-wise lightweight references for global consis- tency constraints and the strengthening of local discrimination. Compared with data-driven attention learning, such regularity priors supply interpretable spatial guidance at low ad- ditional cost, and show potential in indoor scenes where occlusion, scale variation, and competition for high-frequency details coexist [65]. Although existing work offers effective paths for global context, cross-scale fusion, and multi-domain detail restoration, several challenges remain prevalent: first, introducing long-range dependencies often conflicts with preserving 2D topology, and computational and memory costs grow rapidly with scene density; second, cross-level semantic alignment lacks efficiency and stability under long-tailed scales and diverse shapes, and the complexity–benefit trade-off is hard to stan- dardize across platforms; third, systematic approaches to aligning high-frequency detail compensation with task features are still lacking, and frequency enhancement or improved upsampling is sensitive to artifact control and implementation details; fourth, multimodal and 3D solutions face higher barriers in device availability and deployment latency. Based on these shared gaps, subsequent work needs to coordinate global modeling, cross-scale alignment, and detail restoration within a unified and efficient framework, and introduce lightweight and learnable local-regularity priors as spatial guidance, so as to improve robustness and generalization in indoor scenes while maintaining real-time performance. 3. Methodology 3.1. MDF-YOLO Network Architecture Indoor object detection tasks, particularly under severe occlusion, large-scale varia- tions, and dense distributions of small objects, impose higher demands on the multi-level feature modeling capacity of detectors. In recent years, the YOLO family of detectors has become the standard for real-time detection owing to its single-stage design, end-to-end regression paradigm, and well-recognized balance between speed and accuracy. Among them, YOLOv8 features a mature modular design of backbone, neck, and detection head, achieving a strong trade-off between accuracy and inference efficiency on generic bench- marks, while also offering clear advantages in engineering deployment and lightweight extensibility. Consequently, it has been widely adopted as the baseline architecture for subsequent improvements. Despite its competitive performance on large-scale datasets, YOLOv8 still relies heavily on convolutional operations in the backbone, limiting its abil- ity to capture long-range dependencies; its neck fusion strategy lacks a dynamic balance between macro- and micro-contexts; and its up-sampling operators often incur a loss of

Fractal Fract. 2025, 9, 673 8 of 31 high-frequency details during feature recovery. To address these interrelated challenges, we propose Multi-Domain Fusion YOLO (MDF-YOLO), which introduces the CGMB, HG- HCA module, and FGRB into the backbone, neck, and high-resolution recovery stages, respectively. Together, these modules establish complementary feature modeling across the state-space, spatial, and frequency domains. This design enhances global–local collabora- tive reasoning and ensures consistency between structural layouts and fine-grained details, outperforming alternative approaches that rely on a single paradigm by achieving robust and accurate detection under real-time constraints. The overall architecture is illustrated in Figure 1. Input Conv Conv C2f SCB SCB SCB SPPF Concat GCF Detect Head ConcatConcat UpEDFFNGCFUpEDFFN GCF Detect Head Conv Concat GCF Conv Detect Head

Figure 1. Schematic of the MDF-YOLO network architecture. The overall network consists of three components: Backbone, Neck, and Head. The Backbone stage is embedded with the CrossGrid Memory Block to perform long-range dependency modeling in orthogonal directions and extract local convolutional features. The Neck stage introduces the multifractal prior-based context aggregation module, which achieves global–local feature fusion through macro- and micro-pathways guided by regularity. The up-sampling paths incorporate the Frequency-Guided Residual Block, which utilizes a frequency compensation mechanism to recover high-frequency details and fuses them with the main path in a residual form. Finally, multi-scale detection heads perform classification and bounding box prediction. This design achieves complementary feature synergy across the spatial, state-space, and frequency domains, effectively enhancing detection performance in indoor scenes. At the Backbone stage, the network is embedded with the CrossGrid Memory Block (CGMB). This module unfolds the input feature map into one-dimensional sequences along orthogonal directions and employs the Mamba state-space model to achieve long-range dependency modeling, while concurrently retaining a local convolutional path to extract fine-grained textures. The Orthogonal Grid Memory Modeling (OGMM) mechanism can capture complementary pixel relationships in both horizontal and vertical directions and explicitly incorporates spatial information through a position enhancement layer, thereby maintaining semantic consistency and strong spatial awareness at both local and global levels. In the Neck stage, we introduce the Hölder-Based Regularity Guidance–Hierarchical Context Aggregation (HG-HCA) Block. Unlike traditional, purely data-driven fusion meth- ods, the HG-HCA Block is dynamically modulated on a per-pixel basis by a multifractal prior in both its macro-context aggregation path and its micro-context refinement path. Its core lies in the Hölder prior head (HP-head), which performs a lightweight, differentiable approximation of the Hölder exponent on the input features to generate a local regularity map. This map characterizes the smoothness and roughness of different regions and is

Fractal Fract. 2025, 9, 673 9 of 31 transformed into a task-relevant guidance signal via a learnable calibration layer. The macro-path leverages this guidance to strengthen long-range dependency modeling in regular regions, maintaining the consistency of the overall scene layout. Conversely, the micro-path, through directionally sensitive convolution and a dual-attention mechanism, enhances the expression of details in texturally complex regions under the modulation of the fractal prior. Finally, the outputs of both paths undergo deep feature reorganization and residual fusion via a C2f module, achieving a dynamic balance between global and local features. In the up-sampling and high-resolution feature generation stage, the network em- ploys the Frequency-Guided Residual Block (FGRB). This module comprises a spatial up-sampling path and a frequency-domain compensation path. The main path preserves the global structure through bilinear interpolation and convolution. The frequency path maps features to the frequency domain, enhances high-frequency components using a learnable frequency weight matrix, and restores details via transposed convolution, before being finally fused with the main path in a residual manner. This design effectively compen- sates for the high-frequency information loss caused by interpolation during up-sampling, providing a richer feature representation for small objects and texturally complex regions. Through synergistic optimization across the state-space domain (CGMB), the Hölder- based prior-guided spatial domain (HG-HCA), and the frequency domain (FGRB), MDF- YOLO establishes a complementary mechanism for feature modeling and fusion. This enables the network to simultaneously maintain strong robustness and high detection accuracy in complex indoor environments. 3.2. CrossGrid Memory Block In indoor object detection, targets are often subject to severe occlusion and complex stacking, where local textures and geometric boundaries are easily damaged or weakened during feature extraction. This requires detectors to simultaneously capture both local details and long-range dependencies. Traditional convolutional neural networks rely on limited receptive fields and thus face inherent limitations in modeling global spatial re- lationships, often leading to missed or false detections under occlusion and incomplete observations. Existing State Space Models (SSMs) in vision tasks typically flatten the input and map a two-dimensional image into a one-dimensional sequence for process- ing. However, this operation destroys the original spatial topology, making it easy to lose spatial consistency in complex indoor layouts, thereby limiting their applicability in detection tasks. Based on these considerations, we have designed the CrossGrid Memory Block (CGMB), which aims to preserve both the spatial inductive bias of local convolutions and the long-range dependency awareness of state-space modeling, without sacrificing computational efficiency. The core principle of this module is to explicitly maintain the 2D topological structure through an Orthogonal Grid Memory Modeling (OGMM) mechanism, which explicitly maintains two-dimensional topology by unfolding the input features along horizontal and vertical complementary directions. Cosine positional encodings are intro- duced into these sequences to enhance positional information, thereby preserving pixel adjacency and directional consistency on a global scale. Concurrently, a local convolutional path supplements fine-grained features such as textures and edges. This allows the overall representation to encompass both local details and global semantics, providing a more robust feature foundation for subsequent feature fusion and detection. The operational schematic of the CrossGrid Memory Block is depicted in Figure 2.

Fractal Fract. 2025, 9, 673 10 of 31 PE … … PE LN SiLU SSM SiLU 1D Conv Linear Linear Linear LN SiLU SSM SiLU 1D Conv Linear Linear Linear … … Conv Concat Split

Conv ×n Bottleneck Conv Conv Grid Memory Modeling PE or Element-wise Addition Positional Encoding Element-wise Multiplication Bottleneck Conv Conv Conv

Figure 2. Operational schematic of the CrossGrid Memory Block. The CrossGrid Memory Block contains two parallel feature processing branches. The left branch is for Orthogonal Grid Memory Modeling, which captures pixel-level long-range dependencies through modeling in orthogonal directions. The right branch is a local feature extraction path, designed to supplement local texture details. The outputs from the two branches are fused and then combined with a residual connection

Fractal Fract. 2025, 9, 673 11 of 31 and a convolutional module to produce the final output feature. CGMB achieves complementary feature extraction by combining “long-range dependency modeling along orthogonal directions” with “local convolutional detail supplementation,” and their integration under a residual mechanism ensures effective fusion. This design not only preserves strong spatial awareness and structural consistency but also ensures stable gradient propagation during end-to-end training. The entire module receives an input feature map Fin ∈ RC×H×W, where C, H and W denote the number of channels, height, and width of the feature map, respectively. The input is first dispatched into two complementary sub-paths: In the local modeling path (Local Path), the input feature is processed by a lightweight C2f module to extract fine-grained details such as edges and textures: Flocal = C2 f (Fin) (1) In the global modeling path (OGMM Path), we construct a spatial topology-enhanced branch based on state space modeling [ 23]. The input feature map is projected along horizontal and vertical directions into two one-dimensional sequences, Sh and Sv, both of which preserve pixel adjacency in the two-dimensional grid. To avoid the loss of spatial information during unfolding, cosine positional encodings [ 66], denoted as PEh and PEv, are added to each sequence to explicitly embed spatial position information. These sequences are then processed by independent Mamba Blocks to perform selec- tive state space modeling and capture long-range contextual dependencies in their re- spective directions. Finally, the outputs are remapped back to 2D space and fused via element-wise summation: Fglobal = Un f latten [Mamba h(Sh + PEh)] + Un f latten [Mamba v(Sv + PEv)] (2) Compared with the traditional “flatten-and-remodel” approach, this orthogonal fusion strategy preserves linear complexity while avoiding the destruction of two- dimensional topology. After the two aforementioned paths complete their respective modeling, the feature fusion stage integrates the local features Flocal and global features Fglobal through element- wise operations. These are then passed through a residual connection along with the original input Fin to further ensure gradient propagation and information integrity. The fused result is subsequently fed into a standard convolutional module for channel control and non-linear transformation. The final output feature representation of the entire module is given by Fout = Conv  Fin + Flocal + Fglobal  (3) Compared with traditional SSMs that flatten and remodel sequences, CGMB maintains two-dimensional topology. Compared with single-path convolutional or state space model- ing, its dual-path complementarity and residual fusion enhance feature completeness and robustness. Consequently, CGMB demonstrates higher interpretability and adaptability in indoor scenes with severe occlusion and complex stacking, while maintaining strong plug-and-play flexibility and generalizability. 3.3. Hölder-Based Regularity Guidance–Hierarchical Context Aggregation Block Indoor object detection often encounters challenges such as large-scale variations, densely distributed targets, frequent occlusions, and irregular texture and boundary dis- tributions. This requires the detector, at the feature fusion stage, to not only maintain the consistency of global structural information but also emphasize fine-grained features critical for recognizing small objects. Conventional feature pyramid structures can pro-

Fractal Fract. 2025, 9, 673 12 of 31 vide cross-scale alignment, but they lack a dynamic balancing mechanism between global and local contexts. To address this, we propose the Hölder-Based Regularity Guidance– Hierarchical Context Aggregation Block (HG-HCA), which introduces a local regularity map to adaptively modulate the contributions of macro- and micro-context pathways, thereby achieving a self-adjusting balance between global structural modeling and local discriminative enhancement. The overall architecture is illustrated in Figure 3. The core of HG-HCA is the Hölder regularity prior. Given an input feature map Fin ∈ RC×H×W, a lightweight Hölder prior head (HP-head) generates a pixel-wise local regularity map ∼ α(x), which characterizes the smoothness and roughness of local regions. Specifically, three sets of depthwise separable convolutions with different receptive fields are used to simulate multi-scale responses. These responses undergo local energy aggre- gation and logarithmic transformation to approximate wavelet energy spectra, thereby yielding regularity measurements. A 1 × 1 convolution is then applied to linearly combine the multi-scale responses and fit the relationship between scale and energy, producing a regularity map correlated with the Hölder exponent. For stability, the map is further processed with smoothing convolution and instance normalization, followed by a learnable calibration layer to generate a task-related guidance signal: α∗(x) = σ  γ ∼ α(x) − τ  (4) where γ and τ are learnable parameters, and σ(·) denotes the sigmoid activation function. This design allows the network to dynamically learn how the smoothness or roughness of local regions should influence the balance between global and local features in accordance with the task objectives. To enable dynamic adjustment, the calibrated regularity map α∗(x) is further mapped into mutually exclusive weights for the macro and micro pathways. We employ a softmax function to generate the two weights as follows: wmacro (x) = exp(γm(α∗(x) − τm)) exp(γm(α∗(x) − τm)) + exp  γµ  1 − α∗(x) − τµ  (5) wmicro (x) = exp  γµ  1 − α∗(x) − τµ  exp(γm(α∗(x) − τm)) + exp  γµ  1 − α∗(x) − τµ  (6) where (γm, τm) and  γµ, τµ  are two independent sets of learnable parameters. By def- inition, wmacro(x) + wmicro(x) = 1, ensuring mutual exclusivity of the two weights at the pixel level. As a result, pixels in highly regular and smooth regions strengthen the macro-pathway contribution, while those in irregular and texture-rich regions rely more on the micro-pathway, thus achieving a dynamic balance across different spatial areas. In the macro-context pathway, the main objective is to efficiently model long-range dependencies across pixels. To this end, we introduce a reversible reordering operator R(k), which groups and rearranges features along the height or width dimension so that distant